Analysis of Electromagnetic Transmission through Dielectric filled Apertures of Arbitrary Shape in a Thick Conducting Screen

ABUNGU N ODERO1, D B O KONDITI1 AND A V OTIENO2

1Department of Electrical and Electronic Engineering

Jomo Kenyatta University of Agriculture and Technology

P.O. Box 62000, Nairobi

KENYA

2 Department of Electrical and Electronic Engineering

University of Nairobi

KENYA

Abstract – The paper deals with an extension of the previous work appearing in a past issue of this transaction by the authors on hybrid FEM/MOM technique for analyzing transmission properties of arbitrarily shaped apertures on a thick conducting screen. In the present work, the effect of placing different dielectric material slabs in the conducting screen cavity on the electromagnetic transmission parameters is first analyzed and, then, the effect of interchanging the positions of these dielectric slabs relative to the incident field. Validation results for rectangular and cross-shaped slots are presented. Close agreement between our data and published data is observed.  Further data has been generated for rectangular, circular, diamond-shaped and cross-shaped apertures.

Key-Words: – Apertures of arbitrary shape, Dielectric-filled cavity, Different Dieletric slabs at

arbitrary position, Hybrid FEM-MOM technique, Perfect conducting screen

# Introduction

THE FEM technique has been used by many workers to evaluate electromagnetic fields in closed regions [1] – [2 ] due to its ability to accurately model the physical features of an object to a scale much smaller than the wavelength of interest, particularly when dealing with the evaluation of near- field parameters such as , surface currents, input impedance and scattering parameters. The method also handles quite effectively complex inhomogeneities and is less computationally intensive than Finite-Difference Time-Domain (FDTDM) and Moment Method (MOM) because of its sparse and banded matrices.

However, it requires an absorbing boundary for unbounded regions in order to satisfy radiation conditions at far-fields. On the other hand, MOM

technique [3 ] – [ 5], is superior in handling unbounded problems due to its ability to satisfy Sommerfeld radiation conditions at infinity. The FDTM requires a fine subdivision of the computational domain for good resolution and is quite computationally intensive

However, recently, the greatest progress in computational electromagnetics has been in the development and application of hybrid techniques, such as FEM/MOM (finite element-moment method) and FDTM/MOM (finite-difference time-domain-moment method), Xingchao and Yuan [2] and Abungu, et al [6].

# Problem Formulation

Fig.1. illustrates the geometry of the problem under consideration. The configuration consists of a cavity in a thick conducting screen having apertures at the top and bottom surfaces, denoted by Γ1 and Γ2, respectively.

The free space region above and below, z > 0 and z < -d planes are, hereafter, called regions A and B, respectively, and the volume occupied by the cavity (-d< z < 0) referred to as region C. Different layers of material slabs, each of the same thickness, are embedded in region C.

upper aperture

lower aperture

cavity wall

(a)

y

x

o

Region A

(b)

Region C

Region B

conductor

conductor

Ji, Mi

zy

x

o

Fig.1 Problem geometry (a) Top view (b) Cross-sectional

view

To decouple the fields in the three regions, the upper and lower apertures are closed with a perfect electric conductor  (PEC) using an approach similar to that in [7], the tangential electric fields in the aperture planes,Γ1 and Γ2, are replaced with the equivalent magnetic currents M1 in the z = 0+and −M2 in the z = −d− aperture planes, as illustrated in Fig.2.  The problem geometry is then decomposed into 3 regions as illustrated in Fig.3. The magnetic field in regions A and B can be expressed as superposition of the short-circuited magnetic field Hsc, which is the incident magnetic field plus the specular reflection from the PEC plane, and the scattered magnetic field Hscat. On Γ1 and Γ2, the tangential components of the magnetic field can be expressed as

n×Htot=n×(Hsc+Hscat)	              (1)

Region A ( z > 0 )

1

M1(x,y)

-M1(x,y)

x

M2(x,y)

Region C

d

-M22(x,y)

2

Region B ( z < -d )

Γ1

-M1(x,y)

εC, μC

M2(x,y)

Γ2

Fig. 2: Equivalent currents introduced in the aperture planes.

Fig. 3: Problem geometry decomposed into three

regions; A, B, and C.

-M2(x,y)

- z

x  = -d

Γ2

To maintain uniqueness in the solution, the tangential components of the magnetic field must be continuous across Γ1 and Γ2, within region C. Let

Htot(−M1)+Htot(M2) ; Htot(M2)+Htot(−M1) (2)

be the magnetic field on  Γ1 and Γ2,  respectively, produced by the equivalent

currents  -M1 and M2.

By matching the tangential components of the exterior and interior magnetic fields onΓ1 and Γ2, Eq. (1) can be expressed as

n×Hsc=n×(Htot(−M1)+Htot(M2)−Hscat(M1)) on  Γ1

n×Hsc=n×(Htot(M2)+Htot(−M1)−Hscat(M2)) on  Γ2				             (3)

for regions A and B, respectively. Eq. (3) is then solved using the method of moments following a similar procedure as in [5]. The equivalent currents are expanded into a series of known basis functions weighted by unknown coefficients. A set of vector testing functions, which are chosen to be the same as the basis functions (Galerkin procedure), is introduced.

## Computing Exterior Admittance Matrices- MOM

The vector testing functions Wm1 are distributed over Γ1, and the functions Wm2 over Γ2.

Taking the inner product of Eq. (3) with the testing functions and taking the advantage of the linearity of the operators result in on Γ1 given by Eq. (6)

similar expression can be derived for fields on Γ2, with the equivalent magnetic currents M1 and M2 given as

Wm1,H1tsc>=−V1n<Wm1,(H1ttot(−M1n)>+V2n<W1m,H1ttot(M2)>−V1n<Wm1,H1tscat(M1n)>(6)

M1x,y,z=0=n=1NV1nM1nx,y	           	        (7)

M2x,y,z=−d=n=1PV2nM2nx,y	         	        (8)

Now

[YA] = [< W1m, H1tscatMn > ]NxN       		        (9)

2.1.1 Matrix Elements Evaluation

As explained in [9], following Galerkin procedure (Wm = Mm), a typical matrix element for the rth region is given by

Ymn r = 2< Mm, Ht r (Mn) >

= 2Tm + Mm ⋅ Ht r (Mn) ds + Tm − Mm ⋅ Ht r (Mn) ds

=  2Tm ± Mm ⋅Ht r (Mn) ds 	                  (12)

where the notation Tm + ( ) ds  has been introduced for compactness.

In terms of the electric vector potential F (r) and the magnetic scalar potential φ (r), the magnetic field Hr (Mn) can be written as (13)

Applying two-dimensional divergence theorem to

Hr (Mn) = −jω Fn (r) − ∇ φn (r)                           (13)

φn (r) = ∇⋅ Fn (r)−j ω μr εr                            	         (15)

Eq. (12), leads to

Fn (r) = εr TG (rr') ⋅ Mn(r') ds'      	            (14)

Ymn r = − 2jω Tm ±Mm ⋅ Fn ds +2Tm ±mm φn ds                     (16)

Eqn. (16) contains quadruple integrals; a double integral over the field triangles Tm ± and a double integral over the source triangles Tn ± involved in the computation of Fn(r) and φn (r). In order to reduce

[YB] = [< W2m,H2tscatMp > ]PxP                    	 (10)

Ii = [ − W1m,H1tsc ] N×1		   	 (11)

the numerical computations, the integrals over Tm ± can be approximated by the values of integrals at the

centroids of the triangles. This procedure yields

Ymn r = −2lm jω Fn (rmc+)⋅ρmc+2 + Fn (rmc−)⋅ρmc−2       (17)

where	Fn (rm c±) = εr Tn ± G (rc± | r' ) Mn (r') ds'        (18)

φn(rmc±) = −1j ω μr Tn±  ∇ ⋅G (rc± | r' ) ⋅ Mn (r') ds'         (19)

In (17), ρmc± are the local position vectors to the centroids of Tm ± and rmc± = ( rm 1± + rm 2± + rm 3± )  3 are the

position vectors of centroids of Tm ± with respect to the global coordinate system.

FH=e=1NeHe⋅∇×1εr∇×He−k02μrHe⋅HedVe  (24)

Similarly, as explained in [9], using the centroid approximation in Eqn. (11), an element of excitation vector can be written as Im i = −lm  Ht i (rm c+) ⋅ ρm c+2 + Ht i (rm c−) ⋅ρm c−2                (20)

He⋅∇×1εr∇×He=−∇.He×1εr∇×He+1εr∇×He⋅∇×He (25)

## Computing Interior Admittance Matrix YC- FEM

Within the cavity region, the magnetic fields must satisfy the vector Helmholtz equation

FH=e=1Ne1εr∇×He⋅.∇×He−k02μrHe⋅HedVe    	(26)

∇×1εr∇×H−k2μrH=0 	          	 (21)

where H is the total magnetic field. The cavity region is defined as a closed space confined by PEC walls, which will be referred to as the domain V.  A functional described by the inner product of the vector wave equation and the vectors testing function, H, is expressed as

FH=H∗∇×1εr∇×H−k2μrH∗HdV       (22)

is defined. The solution of the boundary-value problem is then found variationally, by solving for the magnetic field at a stationary point of the functional via the first variation

δFH=0				          (23)

The functional has a stationary point (which occurs at an extremum ) and equations (21) and (22) can be used to derive a unique solution for the magnetic field. For an arbitrary volume,V, the solution of the above variational expression cannot be found analytically and is derived via the FEM.  To this end, the domain V is discretized into a finite number of subdomains, or element domains, Ve. Finite elements are chosen with lower degree that are compatible with the required regularity to minimize computational costs. Isoparametric tetrahedron elements (triangles on the surfaces and tetrahedrons for the volumes) with linear basis functions for the integral operators and for the differential surface and volume operators are chosen.

It is assumed that each element domain Ve has a finite volume and that the material profile within this volume is constant.  Within each element domain the vector magnetic field is expressed as He.  If there are Ne element domains within V, the functional can be expressed in terms of the approximate magnetic fields as

Applying the vector identity

and the divergence theorem, (12 ) can be written as

1εrHe.n×∇×He=jk0η0He⋅M                (27)

implying contribution from only regions of implied magnetic current sources, i.e., at the apertures only.

The approximate vector field in each element domain is represented as

H=j=1mαejWej                            	             (28)

with the spatial vector Wejbeing the Whitney basis function defined by

Wij=λi∇λj−λj∇λi                                       (29)

and λi is the barycentric function of node iexpressed as

λir=14+r−rb.Ai3Ve                                        (30)

where Ai is the inwardly directed vectorial area of the tetrahedron face opposite to node i, Ve the element volume, and r the position vector.  Also, rb is the position vector of the barycenter of the tetrahedron defined as

rb=r0+r1+r2+r34                               (31)

in which ri is the position vector of node i.

Using equations (29) in (28) leads to:

FHe=e=1Nej=1mαeji=1mαei1εr∇×Wej.∇×Wei−k2Wej.WeidVe−b=1Nbαbjk0η0Wb.MndS                                  (32)

Letting

Nji=1εr∇×Wej.∇×Wei−k2Wej.WeidVe

(33)

and

Pnb=jk0η0Wb.MndS                           	          (34)

equation (22 ) becomes

FHe=e=1Nej=1mαeji=1mαeiNji−b=1NbαbPnb                    (35)

By enforcing the continuity of the magnetic field, a global number scheme can be introduced.  The following symmetric and highly sparse matrix results

NeeNebNbeNbbαeαb=0Pnb                           (36)

where: αb are coefficients weighting the edge elements lying in the aperture planeΓ1 and Γ2.  αe are coefficients weighting the remaining edge elements in region C.

## 2.2.1 Computing Interior Matrix YC

The computation of the nth column of YC in (6) and (7) is performed by perturbing the cavity with an equivalent current basis function Mn. Then, with the use of (21), the interior tangential magnetic fields in the aperture planes,Γ1 and Γ2, are computed. The first Nrow elements of the nth column of YC are then computed by taking the inner product of the aperture field on Γ1, successively with the N testing functions WN. The next P row elements of the nth column are computed by taking the inner product of the aperture field on Γ2 with the P testing functions WQ. Similarly, the remaining P columns of YC can be computed by perturbing the cavity with equivalent current basis functions Mp.

## Complete Admittance Matrix

The complete admittance matrix from Eq. (13) can be expressed as

The advantage of using this function is that the admittance matrices of the interior and the exterior regions can be constructed independently.

H1scH2sc=Y11CY12CY21CY22CV1nV2n+YA00YBV1nV2n       (37)

The aperture admittance matrices YA and YB are computed by solving the problem of the equivalent magnetic currents radiating into a half-space.  The aperture admittance matrix YC is computed by solving the interior cavity problem.

Figs. 4 and 5 define the dimensions of cross-shaped and diamond-shaped aperture problems analyzed in this work.

# Results and Conclusions

The formulation has first been validated by considering problems of rectangular, circular, diamond-shaped and cross-shaped apertures on thin conducting screens, as given in Table 1, based on the results of Konditi and Sinha [5] and for arbitrarily –shaped apertures on thick conducting screen of Abungu, et al [6] based on hybrid FEM-MOM, respectively. The results are seen to be in good agreement.

Thereafter, circular cross-shaped and diamond-shaped apertures in a screen of varying thickness and having two different dielectric layers filling the cavity have been studied. In each case equivalent magnetic current distributions and transmission cross-sections have been calculated. Relative permittivities for glass, titanium dioxide and air are taken as, 4.0, 96.0, and1.0, respectively.

The results obtained for different commutation of dielectric materials in the cavity region have been presented.  Parameters computed are transmission cross-sections and equivalent magnetic currents.

It is observed in Figs. (6,7,8,9), that the thicker the cavity region the lower the transmission cross-section. Further, it has been established that the transmission cross-sections and magnetic current magnitudes are larger if the incident wave approaches from the higher dielectric constant material toward the lower dielectric constant medium. This could be due to resonance occurring when an EM wave moves from higher dielectric constant material into a lower dielectric constant material. The phase of the equivalent magnetic current M, on the other hand, is independent of the commutation of the dielectric slabs in the cavity region. This phenomenon seems to  be consistent with Floquet’s theorem, which states that given a plane wave incident on a periodic structure, all observable quantities will have the same periodicity as the structure and will have a cell-to-cell phase shift equivalent to that of the periodic structure.

.

References:

- C. M. Butler. Y. Rahmat-Samii. and R. Mittra,. Electromagnetic penetration through apertures in conducting surfaces, IEEE Trans. Electromagn. . vol. EMC-20. pp. 82-93, Feb. 1978.

- Xingchao Yuan, Three-dimensional electromagnetic scattering from inhomogeneous objects by the hybrid moment and the finite element method, IEEE Trans Microwave Theory Tech., Vol. MTT-38, pp. 1053-1058, 1990.

[3]  R.F. Harrington, J.R. Mautz, and D.T. Auckland:

Electromagnetic coupling through apertures,

Dept. Elec. Comput. Eng., Syracuse Univ., Syracuse NY, Rep. TR-81-4, Aug. 1981.

[4] David T. Auckland and Harington, R.F.:

Electromagnetic transmission through a filled slit

in a conducting plane of finite thickness, TE case,

IEEE Microwave Theory Tech., Vol.26, 499-505,

July 1978

[5] Konditi, D.B.O. and Sinha, S. N.:

Electromagnetic Transmission through Apertures

of Arbitrary Shape in a Conducting Screen, IETE

Technical Review, vol. 18, Nos. 2&3, March-

June 2001, pp. 177-190.

[6] Abungu, N. O., Konditi, D.B and Otieno, A.V.:

Hybrid Method Analysis of Electromagnetic

Transmission through Apertures of Arbitrary

Shape in a Thick Conducting Screen, WSEAS

Trans. on Communications, issue 6, vol.3,

December 2004, pp.1767-1778

[7] Harrington, R.F. and Mautz, J.R.: A

generalized network formulation for aperture

problems, IEEE Trans. Antennas Propagat.,

Vol. AP-24, pp. 870-873, 1976

[8] Harrington, R.F. Time–Harmonic

Electromagnetic Fields. New York: McGraw-

Hill, 1961.

[9] Rao, S.M., Wilton, D.R. and Glisson, A.W.:

Electromagnetic scattering by surfaces of

arbitrary shape, IEEE Trans. Antennas

Propagat., Vol. AP-30, pp.409-418, 1982

[10] D.B. Konditi, Moment method analysis of

arbitrary-shape apertures in conducting

screens and waveguides, Ph.D. Thesis, IIT

Roorkee, 2000

Table 1: Validation Data

|  | Konditi & Sinha [3] - MOM Screen Thickness, t = 0.0 |  | Abungu et al [5] Based on Hybrid FEM/MOM Screen thickness, t = 0.001 |  |
| --- | --- | --- | --- | --- |
| # Problem & Dimensions | Peak Equivalent Magnetic Current (M) | Transmission Coefficient | Peak Equivalent Magnetic Current (M) | Transmission Coefficient |
| Rectangular /20 x /2 | 13.4 | 0.33 | 13.2 | 0.32 |
| Cross-shaped (Aw = 2L/3, L = h = 2/3) | 3.3 | 1.3 | 3.1 | 1.2 |
| Diamond-shaped L/ = 0.8 | 22.0 | 0.77 | - | - |
| Circular D/ = 0.5 | 1.25 | 1.65 | 1.15 | 1.55 |

L

h

Aw

x

-x

y

-y

Fig. 4:  Cross-shaped slot dimensions

y

x

L/2

L/2

h/2

h/2

Fig.5:  Diamond-shaped slot dimensions

Fig.6: x-directed Surface Magnetic Current  Distributions at y/h = -0.0625 for Aw = L/4 for Cross-shaped slots in a Conducting Screen of thickness t.

Fig. 7: Transmission Cross-sections y/2 and xx/2 for inhomogeneously filled

Cross-Shaped Slots, L = h = 2/3, Aw = L/4, in a Conducting Screen of thickness t.

Fig. 8: x-directed Surface Magnetic Current Distributions at y/h = -0.0833 for Diamond-shaped slots with

h/ = 0.3556 and L/ = 0.8  in a Conducting Screen of thickness t.

Fig. 9: Transmission Cross-sections y/2 and xx/2 for inhomogeneously filled Diamond-Shaped

Slots, L/ = 0.8, in a Conducting Screen of thickness t.